Tile Setter

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

47-2044.00

RAPIDS CODE

0573

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| Repair and Patch Tile Work |  | 400 |
| Layout Work and Basics |  | 260 |
| Float, Screed, and Scratch Walls as Preparation (Vertical and Horizontal) |  | 1.4K |
| Install all other types of Substrates (i.e., Blue-Board, Dura-Rock and other Moisture Resistant Back-up Materials) |  | 1K |
| Work Walls (Residential and Commercial) |  | 1.4K |
| Prepare and Install Showers and Jambs |  | 840 |
| Prepare and Install Countertops, Pullman, and Sinks |  | 570 |
| Prepare and Install Floors and Steps (All Types) including Precast |  | 680 |
| Work Quarry Tile of all Types in all Applications |  | 490 |
| Prepare and Install Mantels and Hearths Prepare and Install Swimming Pools, Domes, Arches, and Columns |  | 200 |
| Clean, Treat, and Seal Surfaces including Waterproofing and Caulking (All Techniques) |  | 280 |
| Grouting (All Processes) |  | 150 |
| Walls - Thin Set Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install thin set adhering to the following specific criteria: Scrapes, sweeps and cleans wall as required Checks for sound substrate Properly fastened and sound framing conditions per specs. Checks for correct tolerance per specs: Straight Checks for correct tolerance per specs: Flat Checks for correct tolerance per specs: Plumb Checks for painters’ overspray Checks that correct substrate material was used Verifies finish heights Verifies placement, size and shape of accessories and fixtures Identifies any lighting related placement issues Documents and reports any discrepancies to supervisor Follows supervisor’s directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gathers necessary tools and materials Identifies tolerances necessary for the material per the specs Corrects any defects in substrate Verifies corrections by other trades Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes plumb and level controls Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and checks for coverage Adjust and aligns as necessary Cleans work surface Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Walls - Thin Bed Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install thin set adhering to the following specific criteria: Scrapes, sweeps and cleans wall as required Checks for sound substrate Properly fastened and sound framing conditions per specs Properly bonded underlayment’s If contaminants are present, address per specs Checks for correct tolerance per specs: Straight Checks for correct tolerance per specs: Flat Checks for correct tolerance per specs: Plumb Checks for movement (expansion and control) joints If present, verifies location and treats per specs Follows supervisor’s directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gathers necessary tools and materials Identifies tolerances necessary for the material per the specs Corrects any defects in substrate Verifies corrections by other trades Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes plumb and level controls Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and checks for coverage Adjusts and aligns as necessary Cleans work surface Follows supervisor's directions Follows safety rules and requirements appropriate with this section |  |  |
| Floors - Mortar Bed Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install a mortar bed adhering to the following specific criteria: Scrapes, sweeps and cleans work surface Checks for sound substrate Properly fastened and sound framing conditions per specs Properly bonded underlayment's If contaminants are present, address per specs Checks for within tolerance per specs and corrects as necessary - Straight Checks for within tolerance per specs and corrects as necessary - Flat Checks for within tolerance per specs and corrects as necessary - Level Checks for movement (expansion and control) joints If present, verifies location and treats per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gathers necessary tools and materials Identifies tolerances necessary for the material per the specs Corrects-any defects in substrate Verifies corrections by other trades Dampens substrate Applies cementitious bond coat Applies dry pack mortar to wet bond coat Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gathers necessary tools and materials Identifies tolerances necessary for the material per the specs Corrects any defects in substrate Verifies corrections by other trades Applies appropriate cleavage membrane Installs wire reinforcing as required Applies dry pack mortar per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes control and layout lines Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and check for coverage Adjusts and align as necessary Cleans work surface Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Float, Screed and Scratch Walls Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install a flat, screed or scratch wall adhering to the following specific criteria: Scrapes, sweeps and cleans wall Checks for sound substrate Properly fastened and sound framing conditions per specs Checks for within tolerance per specs: Straight Checks for within tolerance per specs: Flat Checks for within tolerance per specs: Plumb Checks for painters' overspray and correct substrate material Verifies finish heights Verifies placement, size and shape of accessories and fixtures Identifies any lighting related placement issues Documents and reports any discrepancies to supervisor Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gathers necessary tools and materials Identifies tolerances necessary for the material per the specs Corrects any defects in framing Verifies corrections by other trades Applies vapor/moisture barrier Applies lath reinforcing Applies scratch coat per specs Applies mortar bed (float coat) per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in framing Verify corrections by other trades Apply vapor/moisture barrier if or as necessary Apply lath reinforcing if or as necessary Apply scratch/bond coat as necessary per specs Apply mortar bed (float coat) per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes control and layout lines Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and check for coverage Adjusts and aligns as necessary Cleans work surface Follow supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Prepare and Install Ceilings Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install a ceiling adhering to the following specific criteria: Scrapes cleans ceiling Checks for sound substrate Properly fastened and sound framing conditions per specs Checks for tolerance per specs: Straight Checks for tolerance per specs: Flat Checks for tolerance per specs: Level or pitched as necessary per specs Checks for painters' overspray and that correct substrate material was used Verifies finish height Verifies placement, size and shape of accessories and fixtures Identifies any lighting related placement issues Documents and reports any discrepancies to supervisor Follow supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in framing Verify corrections by other trades Apply vapor/moisture barrier Apply lath reinforcing Apply scratch coat per specs Apply mortar bed (float coat) per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in framing Verify corrections by other trades Apply vapor/moisture barrier Apply lath reinforcing Apply scratch coat per specs Apply mortar bed (float coat) per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes control and layout lines Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and verifies that coverage and bond are acceptable for overhead installation Adjust and aligns as necessary Cleans work surface Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Prepare and Install Shower Walls and Floors Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install shower walls and floors adhering to the following specific criteria: Scrapes, sweeps and cleans area as required Checks for sound substrate Properly fastened and sound framing conditions per specs Checks for within tolerance per specs: Straight Checks for within tolerance per specs: Flat Checks for within tolerance per specs: Plumb and level Checks for painters' overspray and that correct substrate material was used Verifies finish heights Verifies placement, size and shape of accessories and fixtures Identifies any lighting related placement issues Documents and reports any discrepancies to supervisor Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in framing Checks for proper placement of plumbing Verify corrections by other trades Apply vapor/moisture barrier Applies waterproofing per code Apply lath reinforcing Apply scratch coat per specs Applies mortar bed (float coat) per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in framing Checks for proper placement of plumbing Verify corrections by other trades Apply vapor/moisture barrier Applies waterproofing per code Apply lath reinforcing as necessary Apply scratch or bond coat per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Identify tolerances necessary for the material per the specs Correct any defects in substrate Verify corrections by other trades Inspects drain system and adjust as necessary Dampens substrate Applies cementitious bond coat Applies dry pack mortar to pre sloped specs Allows to cure as necessary Applies waterproofing per specs Protects weep holes Applies wire reinforced mortar bed and slope per specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Verifies adequate slope to drain Establishes control and layout lines Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and checks for coverage Adjusts and aligns as necessary Cleans work surface Follow supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Prepare and Install Countertops, Pullman and Sinks - Thin Set Method Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install countertops, Pullman and sinks adhering to the following specific criteria Scrapes, sweeps and cleans surfaces to be tiled Checks for sound substrate Properly fastened cabinetry and substrate as required Checks for tolerance per specs: Straight Checks for tolerance per specs: Flat Checks for tolerance per specs: Plumb and level Verify corrections by other trades Verifies finish heights and opening widths Verifies placement, size and shape of accessories and fixtures Documents and reports any discrepancies to supervisor Follow supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Protects other finishes Identify tolerances necessary for the material per the specs Installs and/or corrects substrate as necessary Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Protects other finishes Identify tolerances necessary for the material per the specs Installs vapor/moisture barrier to counter and splash per specs Installs wire reinforcing to counter and splash per specs Installs punched metal strip or suitable substitute per specs Sets sink in mortar Applies v,all mortar to splash and counter perimeter per specs Applies dry pack mortar to remainder of countertops Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Follows proper layout according to plans and specs Establishes control and layout lines Spreads bonding material uniformly using correct trowel Installs tile Beats in tile and verifies that coverage and bond are acceptable for overhead installation Adjust and aligns as necessary Cleans work surface and area Follow supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Install Other Types of Substrates - Backer Board Walls Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install substrate backer board walls by adhering to the following specific criteria Checks for sound substrate Properly fastened and sound framing conditions per specs Checks for within tolerance per specs: Straight Checks for within tolerance per specs: Flat Checks for within tolerance per specs: Plumb and level Verifies- finish heights Verifies-placement, size and shape of accessories and fixture openings Identifies any lighting related placement issues Documents and reports any discrepancies to supervisor Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Protects other finishes Identify tolerances necessary for the material per the specs Trues framing as necessary to meet installation tolerance Installs and/or corrects substrate/framing as necessary Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Installs vapor barrier Installs backer board with fasteners per manufacturer's specs and local codes Applies joint treatment per manufacturer's specs Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Install Other Types of Substrates - Backer Board Floors Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install substrate backer board floors by adhering to the following specific criteria Check for sound substrate Properly fastened and sound framing conditions per specs Checks for within tolerance per specs: Straight Checks for within tolerance per specs: Flat Checks for within tolerance per specs: Plumb and level Verifies- finish heights Verifies-placement, size and shape of accessories and fixture openings Documents and reports any discrepancies to supervisor Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Scrapes, sweeps and deans Identify tolerances necessary for the material per the specs Trues existing substrate as necessary to meet installation tolerances Protects other finishes Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Installs backer board with fasteners and adhesives per manufacturer's specs and codes Applies joint treatment per manufacturer's specs Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Self Leveling and Other Cementious Underlayments Provided the necessary and required materials, tools and equipment, the worker must be able to demonstrate without error the ability to install self-leveling- and other cementitious underlayment’s by adhering to the following specific criteria Scrapes, sweeps and deans Checks for sound substrate - Properly fastened and sound framing conditions per specs for wood sub-floors Checks for sound substrate - Clean, dry and free of contaminants for concrete floors per specs Checks for within tolerance per specs: Straight Checks for within tolerance per specs: Flat Checks for within tolerance per specs: Level Verifies finish heights Verifies placement, size and shape of accessories and fixtures Documents and reports any discrepancies to supervisor Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Gather necessary tools and materials Scrapes, sweeps and cleans area Identify tolerances necessary for the material per the specs Protects other finishes Seals all porous areas that may cause material to escape Follows supervisor's directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required Applies primer per manufacturer's specs Mixes and applies underlayment per manufacturer's specs True existing substrate as necessary to meet installation tolerances Cleans work area and tools Follows supervisors directions Follows safety rules and requirements appropriate with this section Follows other processes and procedures as required |  |  |
| Totals |  | OJT HRS 7.7K |

Tile Setter

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

47-2044.00

RAPIDS CODE

0573

| Related Instruction Online | RSI HRS |
| --- | --- |
| Orientation | 12 |
| OSHA Construction Safety 1926 30 hour | 30 |
| CPR/FirstAid | 16 |
| Scaffold Safety Qualification | 6 |
| Drugs and Alcohol | 4 |
| Diversity in the Work Place | 2 |
| Materials Safety Data Sheet | 1 |
| Trade and Labor History | 24 |
| Use of Equipment | 4 |
| Mortars and Adhesives | 32 |
| Use of Hand Tools | 29 |
| Cleaning, Sealing and Finishing | 24 |
| Trade Mathematics | 86 |
| Blueprint Reading | 170 |
| Layout Methodology | 24 |
| Materials Estimating | 34 |
| Industrial and Labor Relations | 48 |
| Advanced Topics/New Technologies | 32 |
| Totals | RSI HRS 580 |