Arborist

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

37-3013.00

RAPIDS CODE

2048

| Work Process Schedule | SKILLS | OJT HRS |
| --- | --- | --- |
| Adhere to ANSI and employer safety standards for all work. Inspect, maintain and use the appropriate personal protective equipment. Inspect, maintain and use the appropriate tool, equipment or vehicle. Assess trees for defects and hazards. Establish and maintain safe work sites and zones. Demonstrate safety practices unique to working aloft. | 5 | 500 |
| Identify common trees and shrubs in the employer’s region. Deciduous Evergreen | 2 | 200 |
| Perform rigging on the ground and aloft. Determine the removal plan. Estimate the maximum load weight based on equipment available. Install and use a rigging system to lower the load directly to the ground. Install and use a rigging system to move the load laterally. | 4 | 400 |
| Climb trees safely. Perform a pre-climb inspection of all equipment. Perform a pre-climb inspection of the tree and site. Install the climbing line(s) from the ground. Ascend the tree. Position to work aloft. Descend the tree. Retrieve the rope and equipment from the ground. | 7 | 1K |
| Operate a chain saw on the ground and aloft, and perform field maintenance. Safely start and control the chain saw. Perform brush cutting. Perform felling. Perform limbing and bucking. Cut wood under tension and compression. Perform removal cuts. Perform pruning cuts. Safely secure and transport the chain saw. Perform field maintenance. | 9 | 800 |
| Operate a chipper. Prepare brush and wood. Position the chipper. Chip brush and wood. | 3 | 300 |
| Prune trees and shrubs from the ground and aloft, according to ANSI A300. Demonstrate awareness of the three reasons for pruning. Determine the pruning objective(s). Perform pruning cuts using the appropriate hand tools and pole tools. Perform crown thinning. Perform crown cleaning. Perform crown reduction. Perform crown raising. Perform structural pruning. Perform pruning of shrubs. | 9 | 1K |
| Perform plant health care activities, including using a motorized spray rig. Plant trees according to ANSI A300. Identify common pests, diseases, and non-infectious disorders in employer's region. Perform spray treatments according to employer practices. Perform trunk injection treatments. Perform soil injection treatments. Perform basal trunk &amp; soil excavation | 6 | 1K |
| Remove trees and shrubs. Grind stumps. | 1 | 800 |
| Local Optional Work Processes Operate vehicles with or without trailer. Operate aerial lifts. Operate stump grinder. Participate in responses to storms and emergency situations. Install tree support and lightning protection systems. | 5 | 560 |
| Totals | SKILLS 51 | OJT HRS 6.6K |

Arborist

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

37-3013.00

RAPIDS CODE

2048

| Related Instruction Online | RSI HRS |
| --- | --- |
| Arborist Skills 1 | 36 |
| Arborist Equipment Fundamentals | 36 |
| Introduction to Aerial Tree Work | 36 |
| Applied Aerial Work for Arborist Apprentices | 36 |
| Tree and Shrub Identification 1 for Arborist Apprentices | 36 |
| Arborist Skills 2 | 36 |
| Rigging and Tree Removal Fundamentals for Arborist Apprentices | 36 |
| Tree Biology & Identification 2 for Arborist Apprentices | 36 |
| Arborist Skills 3 | 36 |
| Advanced Rigging and Tree Felling with Field Skill Building for Arborist Apprentices | 36 |
| Plant Health Care Treatments & Shrub Identification 3 for Arborist Apprentices | 36 |
| Leadership in Tree Care Operations, Communication Skills, and a Capstone Project | 36 |
| Transition to Trainer: Your Role as a Journey Worker | 8 |
| State Pesticide Applicator License, Safety, CPR, First Aid, AED, CDL, and other employer required training or certifications |  |
| Totals | RSI HRS 440 |