Powerplant Mechanic

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-3011.02

RAPIDS CODE

1045

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| General Tasks Calculate and measure capacitance and inductance; calculate and measure electrical power; measure voltage, current, resistance, and continuity; determine the relationship of voltage, current, and resistance in electrical circuits; read and interpret aircraft electrical circuit diagrams, including solid state devices and logic functions; inspect and service batteries. Use aircraft drawings, symbols, and system schematics; draw sketches of repairs and alterations; use blueprint information; use graphs and charts. Weigh aircraft; perform complete weight­and­balance check and record data. Fabricate and install rigid and flexible fluid lines and fittings. Identify and select appropriate non­destructive testing methods; perform dye penetrant, eddy current, ultrasonic, and magnetic particle inspections; perform basic heat­treating processes; identify and select aircraft hardware and materials; inspect and check welds; perform precision measurements. Start, ground operate, move, service, and secure aircraft and identify typical ground operation hazards; identify and select fuels. Identify and select cleaning materials, inspect, identify, remove, and treat aircraft corrosion and perform aircraft cleaning. Extract roots and raise numbers to a given power; determine areas and volumes of various geometrical shapes; solve ratio, proportion, and percentage problems; perform algebraic operations involving addition, subtraction, multiplication, and division of positive and negative numbers. Write descriptions of work performed including aircraft discrepancies and corrective actions using typical aircraft maintenance records; complete required maintenance forms, records, and inspection reports. Use and understand the principles of simple machines; sound, fluid, and heat dynamics; basic aerodynamics; aircraft structures; and theory of flight. Demonstrate ability to read, comprehend, and apply information contained in FAA and manufacturer'’ aircraft maintenance specifications, data sheets, manuals, publications, and related Federal Aviation Regulations, Airworthiness Directives, and Advisory materials, read technical data. Exercise mechanic privileges within the limitations prescribed by FAR 65. Fuels, lubricants, or hydraulic fluids; flammable cements, rosins, sealants, paints and thinners; fluids under pressure; compressed gasses, including oxygen; batteries; aviation ordnance and pyrotechnics; electrical and electronic circuits; operating radio transmitters and radar systems; hazardous noise sources. |  | 1.1K |
| Powerplant Theory and Maintenance Inspect and repair a radial engine; overhaul reciprocating engine; inspect, check, service, and repair reciprocating engines and engine installations; install, troubleshoot, and remove reciprocating engine. Overhaul turbine engine; inspect, check, service, and repair turbine engines and turbine engine installations; install, troubleshoot, and remove turbine engines. Perform powerplant conformity and airworthiness inspections. |  | 600 |
| Powerplant Systems and Components Troubleshoot, service, and repair electrical and mechanical fluid rate­of­flow indicating systems; inspect, check, service, troubleshoot, and repair electrical and mechanical engine temperature, pressure, and R.P.M indicating systems. Inspect, check, service, troubleshoot, and repair engine fire detection and extinguishing systems. Repair engine electrical system components; install, check, and service engine electrical wiring, controls, switches, indicators, and protective devices. Identify and select lubricants; repair engine lubrication system components; inspect, check, service, troubleshoot, and repair engine lubrication systems. Overhaul magneto and ignition harness; inspect service, troubleshoot, and repair reciprocating and turbine engine ignition systems and components; inspect service, troubleshoot, and repair turbine engine electrical starting systems; inspect, service, and troubleshoot turbine engine pneumatic starting systems. Troubleshoot and adjust turbine engine fuel metering systems and electronic engine fuel controls; overhaul carburetor; repair engine fuel metering system components; inspect, check, service, troubleshoot, and repair reciprocating and turbine engine fuel metering systems. Repair engine fuel system components; inspect, check, service, troubleshoot, and repair engine fuel systems. Inspect, check, troubleshoot, service, and repair engine ice and rain control systems; inspect, check, troubleshoot, service, and repair heat exchangers, supercharger and turbine engine airflow and temperature control systems; inspect, check, service, and repair carburetor air intake and induction manifolds. Repair engine cooling system components; inspect, check, troubleshoot, service, and repair engine­cooling systems. Repair engine exhaust system components; inspect, check, troubleshoot, service, and repair engine exhaust systems; troubleshoot and repair engine thrust reverser systems and related components. Inspect, check, service, and repair propeller synchronizing and ice control systems; identify and select propeller lubricants; balance propellers; repair propeller control systems components; inspect, check, service, and repair fixed­pitch, constant­speed, and feathering propellers and propeller governing systems; install, troubleshoot, and remove propellers; repair aluminum alloy propeller blades. Inspect and troubleshoot unducted fan systems and components. Inspect, check, service and troubleshoot turbine­driven auxiliary power units. |  | 1.3K |
| Totals |  | OJT HRS 3K |

Powerplant Mechanic

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-3011.02

RAPIDS CODE

1045

| Related Instruction Online | RSI HRS |
| --- | --- |
| Aviation Machinists Mate Common Core/Hazardous Material Discuss Rules, Regulation and Standard Operating Procedures (SOP) for the Aviation Machinists Mate Common Core. For your Safety Information on Hazardous Material, Material Safety Sheets and Hazardous Materials you may come in contact with. | 8 |
| Physics & Theory Provides the basic foundation in Physics most applicable to Aviation. This lesson will give you the terms and laws of physics, aircraft sections, and the theory and factors affecting flight. | 10 |
| Introduction to Composite Materials You will learn about composite materials, composite structures and structure names, composite materials used on naval aircraft, personnel hazards, and safety precautions associated with composite materials. An understanding of this information is essential to prevent loss of personnel or damage to equipment. | 3 |
| Hydro Mechanical Theory Jet engine maintenance personnel must have an understanding of Hydro mechanical system operation, which will be used in the fuel and lubrication systems of aircraft engines. This lesson will cover the basics needed as to understand components, operation, and safety requirements. | 3 |
| Propulsion Theory Provide you with information on the development of gas turbines, and the terms used in the development of thrust. | 8 |
| Gas­Turbine Engine types and Designations As an aircraft mechanic you are required to know the different types and designation of engines used in military aircraft. You are required to learn and identify the types of engines and the designation numbering system used on engines. | 3 |
| Gas­Turbine Engines The purpose is to provide you with information on the location, designation, and purpose of the different sections and component of the gas­turbine engine. | 13 |
| Basic Electricity The purpose of this information is to familiarize you with the forms of electricity, how it is produced and the terms and symbols used in basic electricity, the electrical diagrams used, what electromagnetic interference (EMI) is, and the electrical test equipment used when dealing with electricity. | 8 |
| Aircraft and Engine Electrical System This information is to familiarize you with the aircraft and engine electrical systems, components, the purpose, and how they function. Safety as always is very important when working with electricity. | 6 |
| Engine Electrical System To provide you with information about engine lubrication system of a jet engine, the types of components used in the lubrication system, types of lubricants, system servicing and proper disposal procedures. An understanding of this information is essential to prevent loss of personnel or damage to equipment. | 6 |
| Aircraft Fuel System To provide you with information on the aircraft fuel system, the types of aviation fuels used the components and their use during fueling, and the methods used in fueling of aircraft. | 4 |
| Engine Fuel System To provide you with information on the engine fuel systems and its components, requirements and purposes. This information will enable you to better understand how the system operates by tracing the path of flow through the system and through each component. | 4 |
| Technical Publications Library Provides information about the types of technical publications used throughout Naval/Marine Corps aviation, the formats in which publications are found, numbering systems used, meaning of the warnings/cautions/notes listed in publications, and the quality assurance programs dealing with technical publications. | 8 |
| Aviation Tools/Hardware To familiarize you with the tool control program as it is utilized throughout Naval/Marine Corps aviation. Identify the use of different types of common and precision tools, and hardware while performing maintenance on aircraft and engines. | 22 |
| Introduction to Corrosion To provide information on the corrosion control program, the theory of corrosion, and materials and procedures for the prevention and repair of corrosion damage to equipment. | 16 |
| Naval Aviation Maintenance Program To provide information about the program that establishes the policies and procedures used throughout Naval/Marine Corps aviation. The inspection/requirements used to maintain aircraft and powerplants in an operational status, the organizations within maintenance departments that maintain the equipment in an operational status and procedures to document aviation maintenance. | 28 |
| Helicopter Powerplants/System Familiarization Provides students with information on helicopter, terms and definitions, and the principle structural units of a helicopter. Flight controls and their location, powerplants, power transmission system, rotary wing systems inspection/troubleshooting, and aircraft cleaning/lubrication. Student performs general and specialized maintenance on Helicopter engines utilizing the appropriate technical manuals while observing safety, FOD prevention and tool control. Documents action taken on VIDS/MAF work order. | 64 |
| Turbojet Powerplants/Systems Familiarization Provides students with information on the types, components, systems, and operation of gasturbine engines. You will perform general and specialized maintenance on jet engines utilizing the appropriate technical manuals while observing safety, FOD prevention and tool control. Documents action taken on VIDS/MAF work order. | 64 |
| Turboprop Powerplants/Systems Familiarization Provides the student with the characteristics, and uses of the T­56 turboprop powerplant, the torque meter assembly, reduction gear assembly, engine oil system, bleed air system, propeller terminology and components, power control rigging and adjustment. You will perform general and specialized maintenance on Turboprop engines utilizing the appropriate technical manuals while observing safety, FOD prevention and tool control. Documents action taken on VIDS/MAF work order. | 64 |
| Totals | RSI HRS 340 |