Geothermal and Well Drilling Operator

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

47-5023.00

RAPIDS CODE

2006

| Work Process Schedule | SKILLS | OJT HRS |
| --- | --- | --- |
| Drill Rig Perform preventive maintenance, proper oils and greases, and minor adjustments. Operate using the controls, their importance in proper operation, and movement of machine for safety of other employees, drilling around underground utilities, working the proper distance from overhead power lines, and other equipment working near the machine. Apply technical knowledge and assist during major overhauls while working with the operator, heavy duty repair person, and welder. “Direct” Mud Rotary: Using both polymer and bentonite based fluids, filtration control, and the use of additives to solve down hole problems. Air Rotary using air | 5 | 3K |
| Excavation Equipment Perform preventive maintenance, proper oils and greases, and minor adjustments. Operate using the controls, their importance in proper operation, and movement of machine for safety of other employees, digging underground utilities, working the proper distance from overhead power lines, and other equipment working near the machine. Perform the movement of heavy drill collars, tooling, as well as digging mud pits and providing drainage to slurry pits. Apply technical knowledge and assist during major overhauls while working with the operator, heavy duty repair person, and welder. | 4 | 500 |
| Tractor-Type Skid Steers Loaders/Hi-Lifts Perform preventive maintenance, proper oils and greases, and minor adjustments. Operate on the different types of work assigned the dozer from rough excavation to finish work. Perform the movement of heavy drill collars, tooling, as well as digging mud pits and providing drainage to slurry pits. Assist in making minor adjustments and repairs and work with the mechanic on major repairs. | 4 | 500 |
| Powered Industrial Trucks Perform preventive maintenance, proper oils and greases, and minor adjustments. Operate on the different types of work assigned the dozer from rough excavation to finish work. Perform the movement of heavy drill collars, tooling, as well as digging mud pits and providing drainage to slurry pits. Assist in making minor adjustments and repairs and work with the mechanic on major repairs. | 4 | 300 |
| Bulldozer Perform preventive maintenance, proper oils and greases, and minor adjustments. Operate on the different types of work assigned the dozer from pioneer and rough excavation to finish work. Assist in making minor adjustments and repairs and work with the mechanic on major repairs. | 3 | 200 |
| Drill Rig Driver Drive a truck mounted drill. Study the Federal Commercial Driver’s License (CDL) laws and obtain a CDL license. | 2 | 350 |
| Use of Grade Instruments and Plans Use instruments and read plans for making grades. Read and set grade stakes as well as read plans and instructions. Read boring plans for layout. | 3 | 450 |
| Material Sampling Perform proper description of soils and logging. Principles and methods of soil collection. Record geological | 3 | 200 |
| Miscellaneous Equipment Operate, service, and adjust all types of pumps. Assist in installing, operating, and maintaining well-point systems. Operate, service, and adjust all types of electric generating plants. Operate, service, and adjust all other types of equipment. | 4 | 300 |
| Maintenance - Cutting and Burning - Greases and Oils Use various welders and welding equipment. Assist in making minor repairs and adjustments. Assist in welding and cutting. Gain knowledge and use appropriate greases and oils. | 4 | 200 |
| Totals | SKILLS 36 | OJT HRS 6K |

Geothermal and Well Drilling Operator

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

47-5023.00

RAPIDS CODE

2006

| Related Instruction Online | RSI HRS |
| --- | --- |
| First Year: Introduction a) Apprenticeship Rules and Regulations b) Issue Books and Manuals c) Drug & Alcohol presentation d) Diversity | 20 |
| First Year: Safety Occupational Safety and Health Administration (O.S.H.A) Rules and Regulations | 10 |
| First Year: Commercial Driver’s License Preparation a) Testing Requirements b) Inspection Stipulations c) Driving Course | 8 |
| First Year: Drilling Fundamentals a) Describe soils b) Log procedures c) Principles and fundamentals | 18 |
| First Year: Health Standard First Aid, CPR and AED | 8 |
| First Year: Basic Equipment Skills (technical training and seat time) a) Drill Rig b) Skid steer loader c) Forklift and Powered Industrial Truck Training d) Support Equipment | 80 |
| Second Year: Soil Exploration a) Theory 1. Syllabi will include American Society for Testing and Materials and American Society of Civil Engineers guidelines where they pertain to geotechnical classes 2. Concentration on the protection of aquifers that provide our civilization its potable water supply. b) Logs and data collection c) Foundations and layout d) Tooling e) Boring | 60 |
| Second Year: Rigging a) Chart Reading b) Drill Set-Up c) Cable Specification | 12 |
| Second Year: Health | 8 |
| Second Year: Basic Equipment Skills (technical training and seat time) | 56 |
| Third Year: Geothermal and other well types a) Theory and components b) Well construction 1. The apprenticeship will make use of the curriculum associated with the International Ground Source Heat Pump Association (IGSHPA) accredited driller classes and will institute the IGSHPA standards and best suggested practices 2. All well head protection standards for the jurisdiction cover through the apprenticeship will be taught and enforced. These may include but are not limited to: i. Bore hole sealing and grouting ii. Grading of the surface around the well head iii. Offset distances for sitting a well from different environmental iv. Hazards v. Residential standards vi. Commercial standards a) Grouting – Mud and Grout Awareness b) Pump selection and installation c) Practical application – 1. Before mixing mud adjust makeup water to the correct ph and calcium content 2. Mix mud to the proper marsh funnel viscosity for the formation being drilled 3. Keep the formation open and/or adjust for down hole problems, clay swelling sand caving etc 4. Take mud weight readings before and during drilling to check solids control of mud tank 5. Drill, log, and sample the hole to a given depth in a given time using safe industry recognized procedures and techniques | 60 |
| Health Standard First Aid, CPR and AED | 8 |
| Use of Grade Instruments and Plans a) Transit and Hand Level b) Leveling Rods c) Measurement Instruments d) Stakes e) Print Reading | 16 |
| Mechanical System a) Basic Hydraulics b) Basic Electrical c) Fundamental Power Trains | 6 |
| Equipment Skills (technical training and seat time) a) Drill Rig – Mud Rotary Proficiency b) Skid steer loader c) Dozer d) Combination Backhoe Loader/Excavator e) Forklift and Powered Industrial Truck f) Support Equipment | 46 |
| Totals | RSI HRS 420 |